DisplayMate for Windows

Gray-Scale Transfer Function Table

This table is calculated using a Gamma Correction Factor of 2.20

It corrects the gray-scale compression for a typical monitor.

Use it to improve color accuracy and photographic gray-scales.

To learn more about this read the Test Information Screen for

the "Gray-Scale Linearity Check" pattern in the Set Up Display

selection, or see the "Gray-Scale Linearity" topic in Windows

Help for DisplayMate.

The table values are not accurate for your particular monitor.

Use the "Color Explore and Match" screen to accurately calibrate

the Gray-Scale Transfer Function values for your own monitor.

Press the "Overlay" button until "Gray-Scale Overlay" appears.

To use the Transfer Function Table to correct the Gray-Scale:

1.  Load it into the video board using a utility provided by

the video board manufacturer. Not all boards have this

capability and each board will have a different procedure.

2.  Some graphics applications programs will allow you to

load it into the program.

3a. Make a list of the RGB intensity "In" values that you need.

b. Replace each intensity "In" value by the "Out" column value.

c. Use the corrected "Out" values in place of the "In" values.

Should you accidentally overwrite this file, a backup copy is

available as "gray_doc.bck" in the DisplayMate directory.

----------------------------------------------------------------

In   Out        In   Out        In   Out        In   Out

----------------------------------------------------------------

0     0   |    64   136   |   128   186   |   192   224

1    21   |    65   137   |   129   187   |   193   225

2    28   |    66   138   |   130   188   |   194   225

3    34   |    67   139   |   131   188   |   195   226

4    39   |    68   140   |   132   189   |   196   226

5    43   |    69   141   |   133   190   |   197   227

6    46   |    70   142   |   134   190   |   198   227

7    50   |    71   143   |   135   191   |   199   228

8    53   |    72   144   |   136   192   |   200   228

9    56   |    73   144   |   137   192   |   201   229

10    59   |    74   145   |   138   193   |   202   229

11    61   |    75   146   |   139   194   |   203   230

12    64   |    76   147   |   140   194   |   204   230

13    66   |    77   148   |   141   195   |   205   231

14    68   |    78   149   |   142   195   |   206   231

15    70   |    79   150   |   143   196   |   207   232

16    72   |    80   151   |   144   197   |   208   232

17    74   |    81   151   |   145   197   |   209   233

18    76   |    82   152   |   146   198   |   210   233

19    78   |    83   153   |   147   199   |   211   234

20    80   |    84   154   |   148   199   |   212   234

21    82   |    85   155   |   149   200   |   213   235

22    84   |    86   156   |   150   200   |   214   235

23    85   |    87   156   |   151   201   |   215   236

24    87   |    88   157   |   152   202   |   216   236

25    89   |    89   158   |   153   202   |   217   237

26    90   |    90   159   |   154   203   |   218   237

27    92   |    91   160   |   155   203   |   219   238

28    93   |    92   160   |   156   204   |   220   238

29    95   |    93   161   |   157   205   |   221   239

30    96   |    94   162   |   158   205   |   222   239

31    98   |    95   163   |   159   206   |   223   240

32    99   |    96   164   |   160   206   |   224   240

33   101   |    97   164   |   161   207   |   225   241

34   102   |    98   165   |   162   207   |   226   241

35   103   |    99   166   |   163   208   |   227   242

36   105   |   100   167   |   164   209   |   228   242

37   106   |   101   167   |   165   209   |   229   243

38   107   |   102   168   |   166   210   |   230   243

39   109   |   103   169   |   167   210   |   231   244

40   110   |   104   170   |   168   211   |   232   244

41   111   |   105   170   |   169   212   |   233   245

42   112   |   106   171   |   170   212   |   234   245

43   114   |   107   172   |   171   213   |   235   246

44   115   |   108   173   |   172   213   |   236   246

45   116   |   109   173   |   173   214   |   237   247

46   117   |   110   174   |   174   214   |   238   247

47   118   |   111   175   |   175   215   |   239   248

48   119   |   112   175   |   176   215   |   240   248

49   120   |   113   176   |   177   216   |   241   249

50   122   |   114   177   |   178   217   |   242   249

51   123   |   115   178   |   179   217   |   243   249

52   124   |   116   178   |   180   218   |   244   250

53   125   |   117   179   |   181   218   |   245   250

54   126   |   118   180   |   182   219   |   246   251

55   127   |   119   180   |   183   219   |   247   251

56   128   |   120   181   |   184   220   |   248   252

57   129   |   121   182   |   185   220   |   249   252

58   130   |   122   182   |   186   221   |   250   253

59   131   |   123   183   |   187   221   |   251   253

60   132   |   124   184   |   188   222   |   252   254

61   133   |   125   184   |   189   223   |   253   254

62   134   |   126   185   |   190   223   |   254   255

63   135   |   127   186   |   191   224   |   255   255