Release Notes

| Program: BF-0505 | Type of Change: Driver (Hardware, firmware, driver, XPA. Etc) Release Date: 02/06/2009 | Revision Number: 1.03.0206’09 Replaced Revision: 1.03.0202’09 Creation Date: 02/13/2008 |
| --- | --- | --- |
| Author or Designer: Sunny_yang |  |  |
| What Changed? |  |  |
| What has been Fixed? | 解決黑白反相無效的問題 |  |
| Known Defects |  |  |

| Required Files And Revision Level | File Name | Revision |
| --- | --- | --- |
|  | BF-0505.dll | 0.0.1.0 |
|  | BF-0505_x64.dll | 0.0.1.64 |
|  | Seconds.ds | 0.1.3.71 |
| Not Changed Files And Version. |  |  |
| Details: |  |  |

Test Overview:

Scanner Driver Info & Test Setup

Enter information about the scanner/driver test.

| Scanner Model Name: | BF-0505 | I/O driver version: |  |
| --- | --- | --- | --- |
| Firmware Version: | 0.0.1.0 | Application: | Photoshop 7.0 |
| TWAIN driver version: | 1.03.0206’09 |  |  |

| # Installer Testing |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Step | Win98 | Win2K | WinXP | WinVista | Comments |
| - Install |  |  |  |  |  |
| - Model name information |  |  | OK |  |  |
| - Scan(200dpi A4 B/W) |  |  | OK |  |  |
| - Uninstall |  |  |  |  |  |
| - Model name information |  |  | OK |  |  |
| - Driver remove |  |  | OK |  |  |
| - Scan Test |  |  |  |  |  |
| - Install driver then reboot PC |  |  | OK |  |  |
| - Scan(200dpi A4 B/W) |  |  | OK |  |  |

| # Basic Functionality Testing |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Step | Win98 | Win2K | WinXP | WinVista | Comments |
| - Mode Test |  |  |  |  |  |
| - Color |  |  | OK |  |  |
| - Gray |  |  | OK |  |  |
| - B/W |  |  | OK |  |  |
| - Brightness Test |  |  |  |  |  |
| - -50 |  |  | OK |  |  |
| - 0 |  |  | OK |  |  |
| - 50 |  |  | OK |  |  |
| - Contrast Test |  |  |  |  |  |
| - -50 |  |  | OK |  |  |
| - 0 |  |  | OK |  |  |
| - 50 |  |  | OK |  |  |
| - Resolution Test |  |  |  |  |  |
| - 100 |  |  | OK |  |  |
| - 150 |  |  | OK |  |  |
| - 200 |  |  | OK |  |  |
| - Simplex\Duplex |  |  |  |  |  |
| - Simplex |  |  | OK |  |  |
| - Duplex |  |  | N/A |  |  |
| - Simulate Mode Test |  |  |  |  |  |
| - A4, 200, B/W |  |  | OK |  |  |
| - A4, 100, Gray |  |  | OK |  |  |
| - A4, 150, Color |  |  | OK |  |  |
| - A4, 200, Gray |  |  | OK |  |  |
| - A4, 150, B/W |  |  | OK |  |  |
| - A4, 200, Color |  |  | OK |  |  |
| - A4, 100, B/W |  |  | OK |  |  |
| - A4, 150, Gray |  |  | OK |  |  |
| - A4, 100, Color |  |  | OK |  |  |
| - Button Test |  |  |  |  |  |
| - Scan button |  |  | OK |  |  |
| - Cancel button |  |  | N/A |  |  |
| - Duplex button |  |  | N/A |  |  |
| - Simplex button |  |  | N/A |  |  |
| - Email button |  |  | OK |  |  |
| - Copy button |  |  | OK |  |  |

| # Special Function Testing |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Step | Win98 | Win2K | WinXP | WinVista | Comments |
| - Hardware Image Processing |  |  |  |  |  |
| - Color drop - None |  |  | OK |  |  |
| - Color drop - R |  |  | OK |  |  |
| - Color drop - G |  |  | OK |  |  |
| - Color drop -B |  |  | OK |  |  |
| - Software Image Processing |  |  |  |  |  |
| - Deskew + Crop (Color) |  |  | OK |  |  |
| - Crop (Gray) |  |  | OK |  |  |
| - Deskew + Crop (B/W) |  |  | OK |  |  |
| - Dynamic Threshold |  |  | OK |  |  |
| - High Quality (Gray) |  |  | OK |  |  |
| - Invert (B/W) |  |  | OK |  |  |
| - Blank Page Detection (Color, Gray, B/W) |  |  | OK |  |  |
| - Rotation (90 Degrees CW) – B/W |  |  | OK |  |  |
| - Rotation (180 Degrees) – Gray |  |  | OK |  |  |
| - Rotation (90 Degrees CCW) – Color |  |  | OK |  |  |
| - Color drop - None |  |  | OK |  |  |
| - Color drop - R |  |  | OK |  |  |
| - Color drop - G |  |  | OK |  |  |
| - Color drop -B |  |  | OK |  |  |
| - Multi-feed Detection |  |  | N/A |  |  |
| - OverScan |  |  | N/A |  |  |
| - Power Saving |  |  | OK |  |  |
| - Mirror |  |  | OK |  |  |
| - Preview (Color, Gray, B/W) |  |  | OK |  |  |
| - Flip Side Rotation (Fanfold) |  |  | OK |  |  |
| - Edge Fill |  |  | OK |  |  |
| - Background Setting (Crop) |  |  | OK |  |  |

| # Adverse Conditions Testing |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Step | Win98 | Win2K | WinXP | WinVista | Comments |
| - Jamming Test |  |  | N/A |  |  |
| - Jam-Release Test |  |  | N/A |  |  |
| - Power Off Test |  |  | N/A |  |  |
| - Cancel Scan Test |  |  | N/A |  |  |
| - ADF Cover Open |  |  | N/A |  |  |
| - Cable disconnect Test |  |  | OK |  |  |