Ⅰ. Brief introduction to USB connect mode:

There are five connect modes for the cell phone. And the cell phone system will pop up the USB Connect Mode window automatically after the USB being connected to PC:

1、After the cell phone is powered on, it enters Phone Tool Mode, and under Phone Tool Mode with CPTone, many operations can be taken such as data sync, cell phone data and software managers, running virtual CD, including cell phone PC driver, e-surfing broadband client and CPTone programs, and adb (factory default mode off);

2、Memory Card Access mode can enumerate sd and udisk to PC and U disk, and meanwhile, the system can run CD and adb;

3、Windows Media Sync mode can sync media documents with PC 10/11 media player and can meanwhile use adb;

4、USB NIC mode compounds adb and rndis devices (used for bundling functionality);

5、Another mode is U-net, which is used with China Telecom U-net Client via dialing.

Ⅱ. Driver Installation

1、To insert the USB and wait for the cell phone virtual CD to install PC driver automatically, and then the system pops up the following Driver Installation window:

2、The program ends automatically after the pre-installation is finished.

Note:

1、If the system does not pop up the above Driver Pre-installation window, PC may prohibit running the CD automatically. Please open My Computer and find out cdrom,

Then open CoolpadInstall virtual CD,

Run the Pre-installation PC Driver in setup.exe under drivers content. Its steps are the same as the above ones.

2、This driver install pack supports win2000, winxp with 32 digits, windows server 20003, vista with 32 and 64 digits and win7 with 32 and 64 digits.

3、If the PC system is windows xp sp1 or sp2 version, the USB NIC is not available. And if you want to use the USB bundling functionality, you have to upgrade your system to sp3 or future versions.

Ⅲ. adb driver installation:

Adb is used to connect to the cell phone debugger on developing. Some PC tools are also connected to cell phone via adb such as 91 assistant and Peasecod. If you want to use adb, you have to turn adb on first.

1、Enter cell phone Settings:

2、Enter Apps:

3、Enter Development

4、Select USB Debug and click OK:

5、If the system is not connected to USB, then it is connected to adb. After being connected to USB, the system installs the driver for adb automatically (however, the pre-installation driver pack on the Second Step has to been executed first), and after it is done, you can find out Coolpad Composite ADB Interface at the Device Manager, then you can use adb functionalities.

Ⅳ. MTP driver installation (just suitable for some Windows XP)

Before installing MTP driver, please make sure that you have installed Media Player 10 or future versions. After installing the required Media Player, if your cell phone is recognized as a Camera (it can be seen at My Computer, as the following picture shows) on being connected to PC, then you need to install the driver according to the following methods:

- To copy yulong_mtp.inf drive document to some content of your PC from drivers/mtp of cell phone virtual CD.

- To right click My Computer, select Manager and then select Device Manager.

- To find out Picture Processor and unfold it to select the corresponding device, and then to take a single click on the right key.

3． Select Update Driver

4． select Install from list or assigned location (advanced)

5． Select Do not search. I want to select my own driver to install.

6.缺少翻译原文

7． And then select Install from disk and find out the copied yulong_mtp.inf at step 1.

8. Select OK.

Finally, select Next to wait to finish installation.

After the installation is done, you can open Windows Media Player to start sync and other operations.

Note: It may take long time to finish the installation for some PC, which contributes to the setting of allowing Network search on your PC.

Ⅴ. USB NIC:

If you want to use USB NIC functionality normally on windows XP, you need to make sure that your system is installed with sp3 or future versions, or you need to upgrade WindowsXP-KB959765-x86-ENU.exe, Microsoft service pack.

USB NIC is mainly used for USB bundling functionality. After USB being bundled, you can treat the cell phone as a wireless modem to surf the Internet on your PC.

Steps:

1、Enter Select USB Mode and select USB NIC Mode;

2、Wait for NIC to install driver automatically, then the system will switch to USB bundling setting interface automatically;

3、Select Bundle USB;

4、If there is another NIC at the lower right corner at PC, it means that it is successful to bundle USB, and then you can surf the Internet via cell phone on your PC.

Ⅵ. Uninstall driver

If you want to uninstall this PC driver after installing it, or there is something wrong with it and you want to tackle the problem via uninstalling it, then you can uninstall it according to the following methods.

1、Connect with USB, open the cell phone virtual CD and enter drivers content:

2 、Run unInstall.exe under drivers content:

3、Wait for the program to be finished and then the system will exist automatically.