iDVR Mobile Surveillance

Version V2.0.9

- Functions

To Use the cell phone monitoring software, users can connect to iDVR series equipment via a mobile wireless network and live view the video on the phone.

Main Functions:

- Device Management

- Edit device properties (IP address or domain, port, user name, password);

- Save the DVR information on the mobile device, so you don’t need to input it again.

- connection and landing equipmen

- After the user completes editing equipment property, enter the connection channel number, then click "Login", the software will automatically try to connect and login the device. If successful, returns the channel's video preview.

- Live view the video

- After successful login, the user can switch the channel to live view the video.

- Video Adjust

- Users preview the surveillance video, you can enlarge the screen on the current channel, increase quality, increase frame rate.

- Remote control PTZ and Camera

- You can remotely control PTZ and camera movement  as you live view the video(but you should have administrator privileges); These actions include PTZ movement (up, down, left, right), to adjust the camera’s lens aperture and focal length, camera closer and farther.

- Performance description

Stream Type: Sub-stream

I frame interval: 2 seconds

Frame Type: Single P frames

Frame rate: 5 frames / sec

Resolution: QCIF (176x144), SCIF (320x240)

- Installation

- Operating Environment

Smart phone / PDA (support CLDC1.0, MIDP2.0 or later)

- Installation Instructions

Open the CellMonitor folder in CD-ROM installation directory, which contains the English version of the cell phone surveillance client and the beta version of the simulator, according to user’s need to copy the corresponding files(CellMonitor.jad and CellMonitor.jar) to your phone in the directory, open CellMonitor. jad file into the installation process.

Select the storage("phone memory" or "expansion card ") to install the software, and then click "Continue." If you select the "expansion card", Please make sure the card is not encrypted, otherwise the phone reboot, the software can not be used.

When the mobile device prompts “installation completed”, you installed the software successfully.

- Help

  - Start the program

Open the software, enter the mobile surveillance system, the main

interface as follows (Fig 1):

Fig 1

Click "Enter" to enter the DVR server login page, as follows (Fig 2):

Fig 2

  - Login DVR Server

Login DVR server settings page, in the "IP address or domain name," enter the DVR server's IP address or domain name (for example, 10376031.dvr-dvs.com, as shown in Fig 3), in the "HTTP Port" enter the DVR server port, The default is 80, "user name" default is admin, default password is empty, in the "connection channel" input the channel number to be connected (Fig 4), click "Login", you can connect to the DVR server and monitor the specified channel;

    - Note: The information of Entering the DVR server is automatically saved on the phone to the next user login;

Fig 3					      Fig 4

Mobile surveillance effect as shown below (Fig 5):

Fig 5

Click "Menu", you can enlarge the monitor screen, switch the channel to preview the video, adjust the image quality, frame rate, and control the camera and PTZ (Fig 6);

Fig 6